User Manual

Advantech ADAM-6700 series

Software API Manual

Document History:

| Doc Version | Date | Comment |
| --- | --- | --- |
| 1.0 | 2019/05/9 | First edition |
| 2.0.1 | 2020/02/05 | Change AdamIO_Open/AdamIO_Close user interface |
| 2.0.2 | 2020/05/26 | Support ADAM-6760 |

Content

1.	Introduction	5

1.1.	About This Manual	5

1.2.	Organization of This Manual	5

2.	User functions	8

2.1.	AdamIO_Open	8

2.2.	AdamIO_Close	8

2.3.	GetModuleName	9

2.4.	GetFirmwareVer	10

2.5.	DIO_GetValues	11

2.6.	DO_SetValue	12

2.7.	DO_SetValues	13

2.8.	GetIOConfigs	14

2.9.	ParseDOConfig	16

2.10.	ParseDIConfig	17

2.11.	SetIOConfigs	17

2.12.	SetDOConfig	19

2.13.	SetDIConfig	21

2.14.	GetDOConfig	22

2.15.	GetDIConfig	23

2.16.	DI_GetDiFilterMiniSignalWidth	24

2.17.	DI_SetDiFilterMiniSignalWidth	26

2.18.	DO_GetPulseOutputCount	27

2.19.	DO_SetPulseOutputCount	28

2.20.	CNT_GetValue	29

2.21.	DO_GetPulseOutputWidthAndDelayTime	30

2.22.	DO_SetPulseOutputWidthAndDelayTime	32

2.23.	ALM_SetLatchClear	33

2.24.	ALM_GetLatchStatus	34

2.25.	CNT_GetStatus	35

2.26.	CNT_SetStatus	36

2.27.	CNT_Clear	37

2.28.	AI_GetFloatValues	38

2.29.	AI_GetFloatValue	40

2.30.	AI_GetInputRange	42

2.31.	AI_SetInputRange	44

2.32.	AI_GetChannelEnabled	45

2.33.	AI_SetChannelEnabled	46

2.34.	AI_SetAutoFilterEnabled	47

2.35.	AI_GetAutoFilterEnabled	48

2.36.	AI_GetAutoFilterSampleRate	49

2.37.	AI_GetBurnoutDetect	50

2.38.	AI_SetBurnoutDetect	52

2.39.	AI_GetBurnoutValue	53

2.40.	AI_SetBurnoutValue	54

2.41.	AI_GetIntegrationMode	55

2.42.	AI_SetIntegrationMode	56

Appendix A	58

Appendix B	60

Chapter	1

- Introduction

  - About This Manual

This is the Software Manual for the Advantech ADAM- 67XX products.

Advantech provides ADAM library which allows developers and end users to connect I/O modules, perform configurations, and simple testing of the I/O.

This manual supplies information about Advantech ADAM-67XX I/O modules, including calling procedure of operating device and descriptions of each function, parameter and data structure.

This manual does not show you how to solve every possible programming problem. To use this manual, you should already be familiar with at least one of the supported programming environments and Linux OS.

  - Organization of This Manual

User functions

This section provides information about user functions and how to use them.

Functions Reference

- Call flow

This section provides call flow and information about how to use functions.

- Error Codes

This section describes system error codes.

Chapter	2

- User functions

  - AdamIO_Open

Users can use this function to open ADAM device. The function returns a handle that can be used to access the ADAM device.

Syntax

AdamIO_Open (

int* pfd

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Output | [out] The driver handler. |

Return Value

If driver initialization succeeded, the return value is valid handle (fd). If the function fails, the return value is Negative integer (<0).

Remarks

Use the AdamIO_Close function to close the ADAM device.

Example

int fd;

AdamIO_Open(&fd);

if(fd<0)

printf( "failed to open i/o\n" );

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AdamIO_Close

Close the ADAM device by calling this function when operation is completed.

Syntax

AdamIO_Close (

int fd

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |

Return Value

The return value is 0.

Example

if(fd>0)

AdamIO_Close (fd);

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - GetModuleName

Users can use this function to get ADAM-67XX module name.

Syntax

GetModuleName (

int fd,

char * o_szName

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| o_szName | Output | [out] The module name. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned int ret=0;

char resp[64] = {};

ret= GetModuleName(fd,&resp[0]);

printf("Module name is %s\n",resp);

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - GetFirmwareVer

Users can use this function to get the firmware version.

Syntax

GetFirmwareVer (

int fd,

char * o_szVer

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| o_szVer | Output | [out] The firmware version. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned int ret=0;

char resp[64] = {};

ret= GetFirmwareVer(fd,&resp[0]);

printf("Firmware version is %s\n",resp);

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - DIO_GetValues

Users can use this function to get all DIO values.

Syntax

DIO_GetValues (

int fd,

int i_iDITotal,

int i_iDOTotal,

unsigned int * o_dwDI,

unsigned int * o_dwDO

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iDITotal | Input | [in] The total channels of DI. |
| i_iDOTotal | Input | [in] The total channels of DO. |
| o_dwDI | Output | [out] The variable to hold the DI values from channel 0 to 31. The LSB indicates the channel 0. |
| o_dwDO | Output | [out] The variable to hold the DO values from channel 0 to 31. The LSB indicates the channel 0. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS) and o_dwDI and o_dwDO contain DIO values from channel 0 to the last channel. If the function fails, please refer the error code definition.

Example

unsigned int dwDI=0, dwDO=0;

ret = DIO_GetValues(fd, iDITotal, iDOTotal, &dwDI, &dwDO);

if(ret == No_Error)

{

printf("DI value in word is 0x%x\n",dwDI);

printf("DO value in word is 0x%x\n\n",dwDO);

for (iCnt = 0; iCnt < iDITotal ; iCnt++)

{

if (dwDI & (0x0001 << iCnt) )

printf("DI Channel %d is true.\n", iCnt);

else

printf("DI Channel %d is false.\n", iCnt);

}

printf("\n");

for (iCnt = 0; iCnt < iDOTotal ; iCnt++)

{

if (dwDO & (0x0001 << iCnt) )

printf("DO Channel %d is true.\n", iCnt);

else

printf("DO Channel %d is false.\n", iCnt);

}

}

else

{

printf("Fail to get DI/DO multiple values, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - DO_SetValue

Users can use this function to set DO value of the indicated slot and channel.

Syntax

DO_SetValue (

int fd,

int i_iChannel,

unsigned char i_bValue

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| i_bValue | Input | [in] The DO value to be set. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

int iChannel = 1;

unsigned char DOValue=0;

ret = DO_SetValue(fd,iChannel, DOValue);

if(ret == No_Error)

{

printf("Succeed to set channel %d DO value: %x\n",iChannel, DOValue);

}

else

{

printf("Fail to set DO value, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - DO_SetValues

Users can use this function to set all DO values.

Syntax

DO_SetValue (

int fd,

int i_iDOTotal,

unsigned int i_dwDO

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iDOTotal | Input | [in] The total channels of DO. |
| i_dwDO | Input | [in] The DO values to be set |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned int dwDO = 0xFFFFFF;

ret = DO_SetValues(fd, iDOTotal, dwDO);

if(ret == No_Error)

{

printf("Succeed to set DO value: 0x%x\n", dwDO);

}

else

{

printf("Fail to set DO value, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - GetIOConfigs

Get DIO configuration: DI/DO mode, record counter enable, difilter enable, invert enable. Use ParseDIConfig and ParseDOConfig to get related configurations.

Syntax

GetIOConfigs (

int fd,

int totalCh,

unsigned char *o_byConfig

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| totalCh | Input | [in] The total channels of DI+DO. |
| o_byConfig | Output | [out] The configuration for all DIO channel |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned char byDIMode=0, byDOMode=0, bRecordLastCount=0, bDigitalFilter=0, bInvert=0;

memset(mode, 0, sizeof(mode));

ret = GetIOConfigs(fd,total,mode);

if(ret == No_Error)

{

for(iCnt=0;iCnt<(TOTAL_DI);iCnt++)

{

ParseDIConfig(mode[iCnt],&byDIMode,&bRecordLastCount,&bDigitalFilter,&bInvert);

printf("DI channel %d mode is %x , record counter enable is %x, difilter enable is %x, invert enable is %x\n",iCnt,byDIMode,bRecordLastCount,bDigitalFilter,bInvert);

}

printf("\n");

for(iCnt=TOTAL_DI;iCnt<(TOTAL_DI+TOTAL_DO);iCnt++)

{

ParseDOConfig(mode[iCnt],&byDOMode);

printf("DO channel %d mode is %x\n",(iCnt-TOTAL_DI),byDOMode);

}

}

else

{

printf("Fail to get configuration of channels, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - ParseDOConfig

Used to parse DO configuration: DO mode

Syntax

ParseDOConfig(

unsigned char i_byConfig,

unsigned char *o_byMode

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| i_byConfig | Input | [in] The configuration of specific channel. |
| o_byMode | Output | [out] The DO mode |

  - ParseDIConfig

Used to parse DI configuration: DI mode, keep last count function, DI filter configuration, DI value invert function.

Syntax

ParseDIConfig(

unsigned char i_byConfig,

unsigned char *o_byMode,

unsigned char *o_bRecordLastCount,

unsigned char *o_bDigitalFilter,

unsigned char *o_bInvert

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| i_byConfig | Input | [in] The configuration of specific channel. |
| o_byMode | Output | [out] The DI mode |
| o_bRecordLastCount | Output | [out] The keep last count function enable/disable |
| o_bDigitalFilter | Output | [out] The DI filter configuration enable/disable |
| o_bInvert | Output | [out] The DI value invert function enable/disable |

  - SetIOConfigs

Used to set all DI/DO channels configuration

Syntax

SetIOConfigs(

int fd,

int totalCh,

unsigned char *i_byConfig

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| totalCh | Input | [in] The total channels of DI+DO. |
| i_byConfig | Input | [in] The configuration for all DIO channel DO mode Value; Meaning. 0; DO. 1; Pulse Output. 2; Low To High Delay. 3; High To Low Delay. DI mode Value; Meaning. 0; DI. 1; Counter. 2; Low To High Latch. 3; High To Low Latch. 4; Frequency. DI configuration mask Value; Meaning. 0xF; DI mode. 0x20; Record Last Count. 0x40; Enable DI Filter. 0x80; Enable Invert Mode. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned char mode[TOTAL_DI+TOTAL_DO]={};

int total = TOTAL_DI+TOTAL_DO;

//index 0~7 is DI, 8~11 is DO

for (iCnt = 0; iCnt < (TOTAL_DI+TOTAL_DO) ; iCnt++)

{

if(iCnt<TOTAL_DI)

mode[iCnt]=DIMode_Di | RECORD_LAST_COUNT | ENABLE_DIGITAL_FILTER | ENABLE_INVERT_MODE;

else

mode[iCnt]=DOMode_Do;

}

ret = SetIOConfigs(fd,total,mode);

if(ret == No_Error)

{

printf("Succeed to configure channels\n");

}

else

{

printf("Fail to configure channels, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - SetDOConfig

Used to set DO channel configuration

Syntax

SetDOConfig(

int fd,

int i_iChannel,

unsigned char i_byConfig);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| i_byConfig | Input | [in] The configuration for DO channel DO mode Value; Meaning. 0; DO. 1; Pulse Output. 2; Low To High Delay. 3; High To Low Delay. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned char i_byConfig;

i_byConfig = DOMode_PulseOutput;

iChannel = 2;

ret = SetDOConfig(fd, iChannel, i_byConfig);

if(ret == No_Error)

{

printf("Succeed to configure channel\n");

}

else

{

printf("Fail to configure channel, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - SetDIConfig

Used to set DI channel configuration

Syntax

SetDIConfig (

int fd,

int i_iChannel,

unsigned char i_byConfig);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| i_byConfig | Input | [in] The configuration for DI channel DI mode Value; Meaning. 0; DI. 1; Counter. 2; Low To High Latch. 3; High To Low Latch. 4; Frequency. DI configuration mask Value; Meaning. 0xF; DI mode. 0x20; Record Last Count. 0x40; Enable DI Filter. 0x80; Enable Invert Mode. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned char i_byConfig;

i_byConfig = DIMode_HighToLowLatch | ENABLE_DIGITAL_FILTER;

iChannel = 3;

ret = SetDIConfig(fd, iChannel, i_byConfig);

if(ret == No_Error)

{

printf("Succeed to configure channel\n");

}

else

{

printf("Fail to configure channel, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - GetDOConfig

Get DO configuration: DO mode. Use ParseDOConfig to get related configurations.

Syntax

GetDOConfig(

int fd,

int i_iChannel,

unsigned char *o_byConfig

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| o_byConfig | Output | [out] The configuration for DO channel |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

ret = GetDOConfig(fd, iChannel, &i_byConfig);

if(ret == No_Error)

{

ParseDOConfig(i_byConfig,&byDOMode);

printf("DO channel %d mode is %x\n",iChannel,byDOMode);

}

else

{

printf("Fail to get configuration of channel, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - GetDIConfig

Get DI configuration: DI mode, record counter enable, difilter enable, invert enable. Use ParseDIConfig to get related configurations.

Syntax

GetDIConfig(

int fd,

int i_iChannel,

unsigned char *o_byConfig

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| o_byConfig | Output | [out] The configuration for DI channel |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

ret = GetDIConfig(fd, iChannel, &i_byConfig);

if(ret == No_Error)

{

ParseDIConfig(i_byConfig,&byDIMode,&bRecordLastCount,&bDigitalFilter,&bInvert);

printf("DI channel %d mode is %x , record counter enable is %x, difilter enable is %x, invert enable is %x\n",iChannel,byDIMode,bRecordLastCount,bDigitalFilter,bInvert);

}

else

{

printf("Fail to get configuration of channel, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - DI_GetDiFilterMiniSignalWidth

Get digital filter minimum signal width

Syntax

DI_GetDiFilterMiniSignalWidth(

int fd,

int i_iChannel,

unsigned int *o_lHigh,

unsigned int *o_lLow

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| o_lHigh | Output | [out] The minimum high signal width |
| o_lLow | Output | [out] The minimum low signal width |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

for(iCnt=0;iCnt<TOTAL_DI;iCnt++)

{

ret = DI_GetDiFilterMiniSignalWidth(fd, iCnt, &high, &low);

if(ret == No_Error)

{

printf("DI channel %d filter low width is 0x%x, high width is 0x%x\n",iCnt,low,high);

}

else

{

printf("Fail to configure DI filter width, error code = %d\n", ret);

}

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - DI_SetDiFilterMiniSignalWidth

Set digital filter minimum signal width

Syntax

DI_SetDiFilterMiniSignalWidth(

int fd,

int i_iChannel,

unsigned int i_lHigh,

unsigned int i_lLow

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| i_lHigh | Input | [in] The minimum high signal width |
| i_lLow | Input | [in] The minimum low signal width |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned int low=0,high=0;

for(iCnt=0;iCnt<TOTAL_DI;iCnt++)

{

high=0x1000;

low=0x1000;

ret = DI_SetDiFilterMiniSignalWidth(fd,iCnt,high,low);

if(ret == No_Error)

{

//printf("Succeed to configure DI filter width\n");

}

else

{

printf("Fail to configure DI filter width, error code = %d\n", ret);

}

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - DO_GetPulseOutputCount

Get pulse output mode and counts.

Syntax

DO_GetPulseOutputCount(

int fd,

int i_iChannel,

unsigned char *o_bContinue,

unsigned int *o_lPulseCount

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| o_bContinue | Output | [out] The pulse output continue mode |
| o_lPulseCount | Output | [out] The pulse output count. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

for(iCnt=0;iCnt<TOTAL_DO;iCnt++)

{

ret = DO_GetPulseOutputCount(fd, iCnt, &Continue, &PulseCount);

if(ret == No_Error)

{

printf("DO channel %d pulse output continue enable is 0x%x, pulse output count is 0x%x\n",iCnt,Continue,PulseCount);

}

else

{

printf("Fail to get DO pulse output count, error code = %d\n", ret);

}

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - DO_SetPulseOutputCount

Set pulse output mode and counts.

Syntax

DO_SetPulseOutputCount(

int fd,

int i_iChannel,

unsigned char i_bContinue,

unsigned int i_lPulseCount

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| i_bContinue | Input | [in] The pulse output continue mode |
| i_lPulseCount | Input | [in] The pulse output count. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned char Continue=0;

unsigned int PulseCount=0;

for(iCnt=0;iCnt<TOTAL_DO;iCnt++)

{

Continue = 0;	//0 is non-continue, 1 is continue

PulseCount = 10000;

ret = DO_SetPulseOutputCount(fd,iCnt,Continue,PulseCount);

if(ret == No_Error)

{

printf("Success to configure DO pulse output count");

}

else

{

printf("Fail to configure DO pulse output count, error code = %d\n", ret);

}

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - CNT_GetValue

Get Counter value.

Syntax

CNT_GetValue (

int fd,

int i_iChannel,

unsigned int *o_lValue

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| o_lValue | Output | [out] The Counter value. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned int CntValue=0;

for(iCnt=0;iCnt<TOTAL_DI;iCnt++)

{

ret = CNT_GetValue(fd, iCnt, &CntValue);

if(ret == No_Error)

{

printf("Counter channel %d value is %lu\n",iCnt,CntValue);

}

else

{

printf("Fail to get counter channel value, error code = %d\n", ret);

}

}

  - DO_GetPulseOutputWidthAndDelayTime

Get pulse output width and delay time.

Syntax

DO_GetPulseOutputWidthAndDelayTime(

int fd,

int i_iChannel,

unsigned int *o_lPulseHighWidth,

unsigned int *o_lPulseLowWidth,

unsigned int *o_lHighToLowDelay,

unsigned int *o_lLowToHighDelay

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| o_lPulseHighWidth | Output | [out] The pulse high signal width |
| o_lPulseLowWidth | Output | [out] The pulse low signal width. |
| o_lHighToLowDelay | Output | [out] The high to low delat time |
| o_lLowToHighDelay | Output | [out] The low to high delay time. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

for(iCnt=0;iCnt<TOTAL_DO;iCnt++)

{

ret = DO_GetPulseOutputWidthAndDelayTime(fd, iCnt, &HighWidth, &LowWidth, &HighDelay, &LowDelay);

if(ret == No_Error)

{

printf("DO channel %d pulse output high width is 0x%x, low width is 0x%x, high delay is 0x%x, low delay is 0x%x\n",iCnt,HighWidth,LowWidth,HighDelay,LowDelay);

}

else

{

printf("Fail to get DO pulse output width and delay time, error code = %d\n", ret);

}

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - DO_SetPulseOutputWidthAndDelayTime

Set pulse output width and delay time.

Syntax

DO_SetPulseOutputWidthAndDelayTime(

int fd,

int i_iChannel,

unsigned int i_lPulseHighWidth,

unsigned int i_lPulseLowWidth,

unsigned int i_lHighToLowDelay,

unsigned int i_lLowToHighDelay

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| i_lPulseHighWidth | Input | [in] The pulse high signal width |
| i_lPulseLowWidth | Input | [in] The pulse low signal width. |
| i_lHighToLowDelay | Input | [in] The high to low delat time |
| i_lLowToHighDelay | Input | [in] The low to high delay time. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned int HighWidth=0,LowWidth=0,HighDelay=0,LowDelay=0;

for(iCnt=0;iCnt<TOTAL_DO;iCnt++)

{

HighWidth=0x1000;

LowWidth=0x1000;

HighDelay=0x1000;

LowDelay=0x1000;

ret = DO_SetPulseOutputWidthAndDelayTime(fd, iCnt, HighWidth, LowWidth, HighDelay, LowDelay);

if(ret == No_Error)

{

}

else

{

printf("Fail to configure DO pulse output width and delay time, error code = %d\n", ret);

}

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - ALM_SetLatchClear

Set the latch alarm to be cleared

Syntax

DO_SetPulseOutputWidthAndDelayTime(

int fd,

int i_iChannel

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

iChannel = 3;

ret = ALM_SetLatchClear(fd,iChannel);

if(ret == No_Error)

{

printf("Succeed to clear channel %d alarm latch\n",iChannel);

}

else

{

printf("Fail to clear alarm latch, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - ALM_GetLatchStatus

Get Alarm Latch Status

Syntax

ALM_GetLatchStatus(

int fd,

int i_iChannel,

unsigned char *o_bLatchStatus

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| o_bLatchStatus | Output | [out] The alarm latch status |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

iChannel = 3;

unsigned char bLatchStatus = 1;

ret = ALM_GetLatchStatus(fd,iChannel,&bLatchStatus);

if(ret == No_Error)

{

printf("Counter channel %d alarm latch status is %x\n",iChannel,bLatchStatus);

}

else

{

printf("Fail to alarm latch status, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - CNT_GetStatus

Get the counter status

Syntax

CNT_GetStatus (

int fd,

int i_iChannel,

unsigned char * o_bCounting

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| o_bCounting | Output | [out] The counter status. True if the counter is counting. Otherwise false. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

for(iCnt=0;iCnt<TOTAL_DI;iCnt++)

{

ret = CNT_GetStatus(fd, iCnt, &Counting);

if(ret == No_Error)

{

printf("Counter channel %d status is %x\n",iCnt,Counting);

}

else

{

printf("Fail to get counter counting status, error code = %d\n", ret);

}

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - CNT_SetStatus

Set the counter status

Syntax

CNT_SetStatus (

int fd,

int i_iChannel,

unsigned char i_bCounting

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |
| i_bCounting | Input | [in] The counter status. True if the counter is counting. Otherwise false. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned char Counting = 1;

for(iCnt=0;iCnt<TOTAL_DI;iCnt++)

{

ret = CNT_SetStatus(fd, iCnt, Counting);

if(ret == No_Error)

{

printf("Success to set counter counting status \n");

}

else

{

printf("Fail to set counter counting status, error code = %d\n", ret);

}

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - CNT_Clear

Clear the counter value

Syntax

CNT_Clear (

int fd,

int i_iChannel

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

iChannel = 3;

ret = CNT_Clear(fd,iChannel);

if(ret == No_Error)

{

printf("Succeed to clear counter channel %d value\n",iChannel);

}

else

{

printf("Fail to clear counter channel value, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\ adam6750io_adam6760io_example.c

  - AI_GetFloatValues

Get the AI float values and channel status.

Syntax

AI_GetFloatValues(

int fd,

int i_iChannelTotal,

float *o_fValues,

unsigned char *o_status

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannelTotal | Input | [in] The total channels of AI |
| o_fValues | Output | [out] The channel value |
| o_status | Output | [out] The channel status Value; Meaning. 0; Normal. 1; Over range. 2; Under range. 3; Burn out. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

float fValues[TOTAL_AI] = {};

unsigned char chstatusAll[TOTAL_AI] = {};

ret = AI_GetFloatValues(fd,iAITotal,&fValues[0],&chstatusAll[0]);

if(ret == No_Error)

{

for(iCnt=0;iCnt<(TOTAL_AI);iCnt++)

{

printf("AI channel %d value is %f, status is %x\n",iCnt,fValues[iCnt],chstatusAll[iCnt]);

}

}

else

{

printf("Fail to AI mutiple float values, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AI_GetFloatValue

Get the AI float value and channel status.

Syntax

AI_GetFloatValue(

int fd,

int i_iChannel,

float *o_fValue,

unsigned char *o_status

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID |
| o_fValue | Output | [out] The channel value |
| o_status | Output | [out] The channel status Value; Meaning. 0; Normal. 1; Over range. 2; Under range. 3; Burn out. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

int iChannel = 1;

float fValue = 0;

unsigned char chstatus = 0;

ret = AI_GetFloatValue(fd,iChannel,&fValue,&chstatus);

if(ret == No_Error)

{

printf("AI channel %d value is %f, status is %x\n",iChannel,fValue,chstatus);

}

else

{

printf("Fail to AI single float values, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AI_GetInputRange

Get the analog input range

Syntax

AI_GetInputRange (

int fd,

int i_iChannel,

unsigned char * o_byRange

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID |
| o_byRange | Output | [out] The input range code Value; Meaning. 0x07; 4~20 mA. 0x08; +/- 10 V. 0x09; +/- 5 V. 0x0A; +/- 1 V. 0x0B; +/- 500 mV. 0x0C; +/- 150 mV. 0x0D; +/- 20 mA. 0x48; 0~10 V. 0x49; 0~5 V. 0x4A; 0~1 V. 0x4B; 0~500 mV. 0x4C; 0~150 mV. 0x4D; 0~20 mA. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

for(iCnt=0;iCnt<TOTAL_AI;iCnt++)

{

unsigned char byRange=0;

ret = AI_GetInputRange(fd, iCnt, &byRange);

if(ret == No_Error)

{

printf("AI channel %d range is 0x%x\n",iCnt,byRange);

}

else

{

printf("Fail to get AI input range, error code = %d\n", ret);

}

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AI_SetInputRange

Get the analog input range

Syntax

AI_SetInputRange (

int fd,

int i_iChannel,

unsigned char i_byRange

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_iChannel | Input | [in] The channel ID |
| i_byRange | Input | [in] The input range code Value; Meaning. 0x07; 4~20 mA. 0x08; +/- 10 V. 0x09; +/- 5 V. 0x0A; +/- 1 V. 0x0B; +/- 500 mV. 0x0C; +/- 150 mV. 0x0D; +/- 20 mA. 0x48; 0~10 V. 0x49; 0~5 V. 0x4A; 0~1 V. 0x4B; 0~500 mV. 0x4C; 0~150 mV. 0x4D; 0~20 mA. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

for(iCnt=0;iCnt<TOTAL_AI;iCnt++)

{

unsigned char byRange=Adam6717_InputRange_V_Neg10To10;

ret = AI_SetInputRange(fd, iCnt, byRange);

if(ret == No_Error)

{

printf("AI channel %d range is 0x%x\n",iCnt,byRange);

}

else

{

printf("Fail to set AI input range, error code = %d\n", ret);

}

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AI_GetChannelEnabled

Get the analog channel enable/disable

Syntax

AI_GetChannelEnabled (

int fd,

unsigned char *o_byEnabledMask

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| o_byEnabledMask | Output | [out] The channel mask |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned char byEnabledMask = 0;

ret = AI_GetChannelEnabled(fd,&byEnabledMask);

if(ret == No_Error)

{

printf("AI channel enable/disable mask is 0x%x\n",byEnabledMask);

}

else

{

printf("Fail to get AI channel enable/disable mask, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AI_SetChannelEnabled

Set the analog channel enable/disable

Syntax

AI_SetChannelEnabled (

int fd,

unsigned char i_byEnabledMask

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_byEnabledMask | Input | [in] The channel mask |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

byEnabledMask = 0xFF;

ret = AI_SetChannelEnabled(fd,byEnabledMask);

if(ret == No_Error)

{

printf("AI channel enable/disable mask is 0x%x\n",byEnabledMask);

}

else

{

printf("Fail to set AI channel enable/disable mask, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AI_SetAutoFilterEnabled

Set the AI auto filter enable mask and the filter rate of FSR.

Syntax

AI_SetAutoFilterEnabled(

int fd,

unsigned char i_byFilterEnabledMask,

int i_iPercentIndex);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_byFilterEnabledMask | Input | [in] The auto filter enable mask |
| i_iPercentIndex | Input | [in] The filter rate of FSR |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

FilterEnabledMask = 0xFF;

PercentIndex = 7;	//The percentage index. 1 means 10%, ..., 9 means 90%.

ret = AI_SetAutoFilterEnabled(fd,FilterEnabledMask, PercentIndex );

if(ret == No_Error)

{

printf("AI Auto-filter enable/disable mask is 0x%x, filter rate is %d%\n",FilterEnabledMask,PercentIndex*10);

}

else

{

printf("Fail to set Auto-filter enable/disable mask and filter rate, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AI_GetAutoFilterEnabled

Get the AI auto filter enable mask and the filter rate of FSR.

Syntax

AI_GetAutoFilterEnabled(

int fd,

unsigned char *o_byFilterEnabledMask,

int *o_iPercentIndex);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| o_byFilterEnabledMask | Output | [out] The auto filter enable mask |
| o_iPercentIndex | Output | [out] The filter rate of FSR |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned char FilterEnabledMask = 0;

int PercentIndex = 0; //The percentage index. 1 means 10%, ..., 9 means 90%.

ret = AI_GetAutoFilterEnabled(fd,&FilterEnabledMask, &PercentIndex );

if(ret == No_Error)

{

printf("AI Auto-filter enable/disable mask is 0x%x, filter rate is %d%\n",FilterEnabledMask,PercentIndex*10);

}

else

{

printf("Fail to get Auto-filter enable/disable mask and filter rate, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AI_GetAutoFilterSampleRate

Get the AI auto filter sample rate.

Syntax

AI_GetAutoFilterSampleRate (

int fd,

int *o_iRate);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| o_iRate | Output | [out] The sample rate |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

int iRate = 0;

ret = AI_GetAutoFilterSampleRate(fd,&iRate);

if(ret == No_Error)

{

printf("Auto-filter sampling rate is %d\n",iRate);

}

else

{

printf("Fail to get Auto-filter sampling rate, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AI_GetBurnoutDetect

Get the AI burnout detect mode enable/disable

Syntax

AI_GetBurnoutDetect (

int fd,

unsigned char * o_burnout

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| o_burnout | Output | [out] The burnout detect mode |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned char burnout = 0;

ret = AI_GetBurnoutDetect(fd,&burnout);

if(ret == No_Error)

{

printf("AI burnout detect enable/disable is 0x%x\n",burnout);

}

else

{

printf("Fail to get AI burnout detect enable/disable, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AI_SetBurnoutDetect

Set the AI burnout detect mode enable/disable

Syntax

AI_SetBurnoutDetect (

int fd,

unsigned char i_burnout

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_burnout | Input | [in] The burnout detect mode |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

burnout = 1;

ret = AI_SetBurnoutDetect(fd,burnout );

if(ret == No_Error)

{

printf("AI burnout detect enable/disable is 0x%x\n",burnout);

}

else

{

printf("Fail to set AI burnout detect enable/disable, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AI_GetBurnoutValue

Get the AI burnout value

Syntax

AI_GetBurnoutValue (

int fd,

unsigned char * o_burnoutValue

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| o_burnoutValue | Output | [out] The burnout value. 0 is down scale, 1 is up scale. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned char burnoutValue = 0;

ret = AI_GetBurnoutValue(fd,&burnoutValue);

if(ret == No_Error)

{

printf("AI burnout value is 0x%x\n",burnoutValue);

}

else

{

printf("Fail to get AI burnout value, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AI_SetBurnoutValue

Set the AI burnout value

Syntax

AI_SetBurnoutValue (

int fd,

unsigned char i_burnoutValue

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_burnoutValue | Input | [in] The burnout value. 0 is down scale, 1 is up scale. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

burnoutValue = 1;

ret = AI_SetBurnoutValue(fd,burnoutValue );

if(ret == No_Error)

{

printf("AI burnout value is 0x%x\n",burnoutValue);

}

else

{

printf("Fail to set AI burnout value, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AI_GetIntegrationMode

Get the AI integration mode

Syntax

AI_GetIntegrationMode (

int fd,

unsigned char * o_mode

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| o_mode | Output | [out] The integration mode Value; Meaning. 0x0; 50/60 Hz. 0x80; User defined. 0xA0; High speed. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

unsigned char integrationMode = 0;

ret = AI_GetIntegrationMode(fd,&integrationMode);

if(ret == No_Error)

{

printf("AI Integration Mode is 0x%x\n",integrationMode);

}

else

{

printf("Fail to get AI Integration Mode, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

  - AI_SetIntegrationMode

Set the AI integration mode

Syntax

AI_SetIntegrationMode (

int fd,

unsigned char i_mode

);

Parameters

| Name | Direction | Description |
| --- | --- | --- |
| fd | Input | [in] The driver handler. |
| i_mode | Input | [in] The integration mode Value; Meaning. 0x0; 50/60 Hz. 0x80; User defined. 0xA0; High speed. |

Return Value

If the function succeeds, the return value is 0 (ERR_SUCCESS). If the function fails, please refer the error code definition.

Example

integrationMode = MODE_HIGH_SPEED;

ret = AI_SetIntegrationMode(fd,integrationMode );

if(ret == No_Error)

{

printf("AI Integration Mode is 0x%x\n",integrationMode);

}

else

{

printf("Fail to set AI Integration Mode, error code = %d\n", ret);

}

For more detailed information regarding this function, please see

$(SDK directory)\IO\adam6717io_example.c

# Appendix A

Call flow

Users can directly access drivers with APIs.

Common Call Flow

The following figure describes the common call flow of the ADAM- 67XX products.

Start

Open ADAM Device

By using AdamIO_Open

Device Handle

Get/set channel information

By using specific functions

Device Handle

AI/AO modules: See the function name starting with AI, AIO, AO.

Counter modules: See the function name starting with CNT.

DI/DO modules: See the function name starting with DI, DIO, DO.

AI/AO modules: See the function name starting with AI, AIO, AO.

Counter modules: See the function name starting with CNT.

DI/DO modules: See the function name starting with DI, DIO, DO.

Close ADAM Device

By using AdamIO_Close

End

# Appendix B

Error Codes

The information on this page is intended to be used by programmers so that the software they write can better deal with errors.

The following list describes system error codes.

No_Error

0x00000000

No error.

ComPort_Error

0x40000001

Serial port error.

ComPort_Open_Fail

0x40000002

The system fails to open serial port.

ComPort_Send_Fail

0x40000003

Send data to serial port fail.

ComPort_Recv_Fail

0x40000004

Receive data from serial port fail

Adam_Invalid_Head

0x40080001

Adam protocol response header is invalid.

Adam_Invalid_End

0x40080002

Adam protocol response header is invalid.

Adam_Invalid_Length

0x40080003

Adam protocol response length is invalid.

Adam_Invalid_Data

0x40080004

Adam protocol response data is invalid, which causes the convertion failed.

Adam_Invalid_Checksum

0x40080005

Adam protocol response checksum is invalid.

Adam_Invalid_Param

0x40080006

The parameter is invalid.

Adam_Invalid_Password

0x40080007

The password is invalid.

API_Parameter_Error

0x400B0003

API parameter error.